Airspace Assessment / Hazard Identification

Identification of potential other airspace users in the intended volume of operation and adjacent airspace, and whether they are cooperative.

Based on EUROCONTROL Unmanned Aircraft Systems – ATM Collision Avoidance Requirements , §5.4.7, Table 7 :

| Aerial objects by group |  |  | Hazard Identification |  |  | Risk Characterization |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cat | Group | Description | Considered as Hazard [yes / no] | Equippage Conspicuity | Traffic Characteristics Rationale to not consider it as Hazard Hazard characterization within airspace and details on conspicuity requirements | Strategic Mitigations (included in SORA) *Soft factors (excluded from the SORA scope) | Tactical Mitigations (included in SORA) *Soft factors (excluded from the SORA scope |
| 1 | F | Fauna. Large Birds | No | None | (e.g.) Impact on Ground Risk only |  |  |
|  | K | Kites and tethered balloons. |  |  |  |  |  |
|  | B | Hot air balloons. | No | MODE-S (a) | - Balloons tend to operate in known weather and areas. - Traffic exposure at low level is limited in airport environment and controlled airspace - For gas and hot air balloons within airspace C and D, an authorisation is required from the appropriate air traffic control units. - Minimum VFR height (ie 500ft/AGL) from VRV-L/SERA are applicable to balloons (except for training) and main balloons sites are visible on the ICAO chart for flight planning. - (a) Transponder MODE-S only required for flights during aeronautical night and dense fog (VRV-L, art. 29) | - Flight planning, identification of the flight time and areas for B Ops - Monitoring of radio communication Soft factors Balloons are identifiable on on-board camera / ground-based webcams due to their size | - Transponder MODE-S detection through MLAT Soft factors Balloons are identifiable on on-board camera due to their size |
|  | P | Parachutists |  |  |  |  |  |
|  | A | Unpowered air sports: hang gliders, paragliders, etc. |  |  |  |  |  |
| 2 | R | Radio controlled model aircraft |  |  |  |  |  |
|  | G | Gliders. |  |  |  |  |  |
|  | S | Powered air sports: very light aircraft, ultralights, motor gliders, motor paragliders, etc |  |  |  |  |  |
| 3 | D | Dirigible airships |  |  |  |  |  |
| 4 | H-State | Helicopters (VIP, Police, Customs, Civil Protection, Military) |  |  |  |  |  |
|  | H-SAR | Helicopters (emergency and medical services SAR/HEMS) |  |  |  |  |  |
|  | H-CAT | Commercial rotorcraft (CAT) incl. SPO (Aerial Work) |  |  |  |  |  |
|  | H-GAR | General aviation rotorcraft (GAR) |  |  |  |  |  |
|  | L | Light aircraft (i.e. non-pressurised general aviation). |  |  |  |  |  |
|  | Q | Pressurised general aviation with MTOM less than 5,700 kg. |  |  |  |  |  |
|  | N | Pressurised pax acft no ACAS |  |  |  |  |  |
| 4,5 | T | Pressurised pax acft w/ ACAS |  |  |  |  |  |
|  | C | Cargo aircraft or military air transport. |  |  |  |  |  |
|  | M | Military trainer aircraft and high-performance jets. |  |  |  |  |  |
|  | U | Unmanned Aircraft |  |  | Out of SORA Scope |  |  |

Table  1: Airspace Assessment - Aerial Objects by group

Unmanned Aircraft Systems – ATM Collision Avoidance Requirements ... EUROCONTROL, UAS ATM Integration Activity, version 1.0,. November 2008, https://skybrary.aero/sites/default/files/bookshelf/3208.pdf [online link]